# Project Report: PWM Control using 555 Timer

## 1. Title

Pulse Width Modulation (PWM) using 555 Timer

## 2. Aim

To design and develop a circuit that generates PWM using a 555 timer IC.

## 3. Introduction

PWM is used to control power delivered to electronic devices. A 555 timer in astable mode can generate a PWM signal by adjusting charge and discharge times.

## 4. Components Required

- 555 Timer IC

- Potentiometer 100kΩ

- Capacitor 10nF

- Resistor 10kΩ

- Diodes (1N4148) x2

- Breadboard & wires

- 5-12V DC supply

## 5. Circuit Description

The circuit uses diodes to separate charge and discharge paths, giving adjustable duty cycle while frequency stays almost constant.

## 6. Procedure

1. Place 555 IC on breadboard.

2. Connect Vcc and GND.

3. Connect pins 2 & 6 together with capacitor.

4. Add potentiometer, diodes, resistor as per circuit.

5. Apply DC supply.

6. Observe PWM output at pin 3.

## 7. Observations

Duty cycle changes from ~5% to ~95% with potentiometer adjustment.

## 8. Conclusion

PWM was successfully generated using a 555 timer. Duty cycle was adjustable while frequency remained stable.