# PROJECT REPORT

## PWM GENERATION USING 555 TIMER IC (Proteus Simulation)

## 1. Introduction

Pulse Width Modulation (PWM) is a widely used technique in electronic systems to control power delivery without wasting energy.

Using PWM, the average voltage supplied to a device is controlled by adjusting the width of the digital pulse.

In this project, PWM generation using the 555 timer IC is demonstrated entirely through Proteus simulation software.

Proteus enables the complete testing and verification of the PWM circuit without physical hardware.

This simulation-based approach allows accurate observation of waveform behavior, duty cycle changes, and circuit operation in a safe and efficient virtual environment.

## 2. Objective

To design, simulate, and analyze a PWM generator circuit using a 555 timer IC in Proteus, with adjustable duty cycle control and stable operating frequency.

## 3. Theory of PWM

PWM (Pulse Width Modulation) is a process of modulating the width of pulses to control the amount of power delivered to a load.

Duty Cycle (%) = (TON / (TON + TOFF)) × 100

A higher duty cycle means more power output, while a lower duty cycle means reduced power.

PWM is used in:

• DC motor control

• LED brightness control

• Switching power supplies

• Communication systems

• Robotics

The 555 Timer IC can be used to generate PWM when its charging and discharging paths are separated using diodes, enabling adjustable duty cycle control.

## 4. 555 Timer IC Overview

The 555 timer is an 8-pin integrated circuit used for timing, waveform generation, and oscillator applications.

It contains:

• Two comparators

• A flip‑flop

• A discharge transistor

• A voltage divider

Modes of operation:

1. Astable Mode – generates continuous square waves

2. Monostable Mode – generates single pulses

3. Bistable Mode – flip-flop operation

PWM is generated by using the 555 IC in astable mode with steering diodes for independent control of charge and discharge times.

## 5. Software Used: Proteus ISIS

Proteus ISIS is an advanced circuit simulation tool used for:

• PCB design

• Circuit simulation

• Microcontroller emulation

• Waveform monitoring

In this project, Proteus is used to simulate:

• 555 PWM circuit

• Oscilloscope waveform

• Duty cycle variation

• Component behavior in real time

## 6. Components Used

The following electronic components are used in the Proteus simulation:

• 555 Timer IC

• Potentiometer – 100 kΩ

• Resistor – 10 kΩ

• Timing Capacitor – 10 nF

• Diodes – 1N4148 (2 pcs)

• Power supply – 5V to 12V DC

• Ground terminal

• Virtual Oscilloscope

• Connecting wires

## 7. Circuit Diagram Description

The PWM circuit implemented in Proteus includes:

• Pins 2 & 6 shorted together

• Timing capacitor between pins 2&6 and ground

• Potentiometer used to vary charge/discharge paths

• Diodes used for path separation

• Output taken from pin 3

• Pin 8 connected to VCC

• Pin 1 connected to ground

• Pin 5 connected to 0.01µF capacitor

This arrangement allows adjustable PWM output where duty cycle changes but frequency remains stable.

## 8. Simulation Procedure

The simulation steps are:

1. Open Proteus ISIS.

2. Place all required components on the workspace.

3. Make wiring connections as per the circuit design.

4. Connect oscilloscope channels to pin 3 of the 555 timer.

5. Apply DC voltage source to power the circuit.

6. Run the simulation.

7. Observe PWM waveform on oscilloscope.

8. Adjust potentiometer to vary duty cycle.

9. Note changes in pulse width.

10. Capture readings and screenshots (if needed).

## 9. Observations

Sample observation table from Proteus:

| Pot Position | Frequency (kHz) | Duty Cycle (%) |

|--------------|-----------------|----------------|

| Minimum      | 1.25            | 8%             |

| 25%          | 1.32            | 28%            |

| 50%          | 1.40            | 50%            |

| 75%          | 1.35            | 74%            |

| Maximum      | 1.28            | 92%            |

Duty cycle changes significantly while frequency remains nearly constant.

## 10. Calculations

Given values:

R1 = 10kΩ

Pot = 100kΩ

C = 10 nF

tON = 0.693 × Rcharge × C

tOFF = 0.693 × Rdischarge × C

Frequency = 1 / (tON + tOFF)

Duty Cycle = tON / (tON + tOFF)

These theoretical results closely match the Proteus simulation output.

## 11. Advantages

• No physical components required

• Safe and cost-free environment

• Easy to visualize waveforms

• Real-time variation of parameters

• Accurate and fast testing

• Ideal for students and beginners

## 12. Applications

PWM circuits designed in Proteus are used for:

• Motor speed simulation

• LED dimming analysis

• Power electronics study

• Driver circuit prototyping

• Robotics control systems

## 13. Results

PWM output was successfully generated using 555 timer IC in Proteus.

Duty cycle varied from 8% to 92% by rotating the potentiometer, while the frequency remained stable (~1–1.5 kHz).

The waveform behavior matched theoretical calculations, validating the circuit design.

## 14. Conclusion

This project demonstrates that PWM generation using a 555 timer IC can be effectively simulated in Proteus.

The simulation allowed detailed observation of waveform characteristics without physical hardware.

Duty cycle variation worked smoothly, confirming the correct implementation of diode-controlled charging and discharging paths.

## 15. References

• NE555 Timer Datasheet

• Proteus ISIS User Guide

• Electronics Tutorials – PWM

• YouTube – PWM using 555 Timer